## No. 15                                                                                             28 April 2003 – 08:00 UTC

BEPPOSAX REENTRY PREDICTIONS

Prepared by Luciano Anselmo and Carmen Pardini

Space Flight Dynamics Laboratory

Istituto di Scienza e Tecnologie dell’Informazione “Alessandro Faedo”

CNR - Area della Ricerca di Pisa, Via Moruzzi 1, 56124 Pisa, Italy

ASI Contract No. I/408/02/0

BeppoSAX Reentry Situation Room: bepposax@protezionecivile.it

tel. +39-06-6804-3255 ~ fax +39-06-6889-7689

# SATELLITE ORBIT STATUS

Currently BeppoSAX is approaching the reentry into the densest layers of the earth atmosphere. The fragments of the satellite able to survive the atmospheric reentry might still reach in principle any location inside the equatorial belt in between 4.36 degrees North and South. The probability to hit a person in the open is 4.5 × 10-4, while the chance to strike a commercial aircraft in flight is of the order of 10-7. On the other hand, about 150-200 tons of orbital objects reenter the earth atmosphere every year without control. For instance, two upper stages (one Russian and one American) decayed during the last two days, and another one (European) will reenter in the coming 48 hours.

In the last 24 hours, it will be possible to exclude from the risk substantial areas of the above-mentioned equatorial belt, while maps and impact time windows will be provided for the regions potentially interested by the debris fall. However, a rough estimate of the reentry location, with an uncertainty of several thousand kilometers along the final trajectory, will be available only some hours after the fact, unless reliable and consistent visual observations are reported.

# BALLISTIC PARAMETER ESTIMATION

The ballistic parameter B, defined according to the relationship B  CDA/m, where CD, m and A are, respectively, the satellite drag coefficient, mass and average cross-sectional area, was estimated by fitting, in a least squares sense, the satellite semi-major axis decay observed during the last two days. All the relevant orbit perturbations were included and the Jacchia-Roberts 1971 model was adopted to describe the varying atmospheric density. The orbital data were provided by the NASA/GSFC Orbital Information Group and by Nicholas L. Johnson of NASA/JSC. The value obtained was B  0.02471 m2/kg.

# BEPPOSAX DECAY WINDOW

The expected residual lifetime of the satellite was computed by propagating its last available state vector (epoch: 27 April 2003, 22:40 UTC) with a numerical trajectory predictor including the geopotential harmonics up to the 16th order and degree, the luni-solar attraction, the solar radiation pressure with eclipses and the aerodynamic drag. The air density was computed with the Jacchia-Roberts 1971 model.

Concerning the predicted solar flux at 10.7 cm (used by the atmospheric model as a proxy of the solar irradiance at extreme ultraviolet frequencies) and the geomagnetic planetary index Ap, the last 45-day forecast, prepared by the U.S. Air Force and issued by the NOAA Space Environment Center, was adopted.

Using for BeppoSAX the ballistic parameter estimated by fitting the altitude decay observed during the last two days, the ground impact uncertainty window shown in Table 1 was obtained for the main debris. The confidence level associated with such a window is approximately 90%.

Table 1

Ground Impact Time Window for the BeppoSAX Main Debris

| Impact Window | Impact Date | Impact Time |
| --- | --- | --- |
| Opening | 29 April 2003 | 16:47 UTC |
| Nominal Impact | 30 April 2003 | 03:08 UTC |
| Closure | 30 April 2003 | 20:16 UTC |

# NEXT UPDATE

The next reentry prediction will be issued on 28 April 2003, as soon as new two-line orbital elements will be available.

