## No. 23                                                                                             29 April 2003 – 21:40 UTC

BEPPOSAX REENTRY PREDICTIONS

Prepared by Luciano Anselmo and Carmen Pardini

Space Flight Dynamics Laboratory

Istituto di Scienza e Tecnologie dell’Informazione “Alessandro Faedo”

CNR - Area della Ricerca di Pisa, Via Moruzzi 1, 56124 Pisa, Italy

ASI Contract No. I/408/02/0

BeppoSAX Reentry Situation Room: bepposax@protezionecivile.it

tel. +39-06-6804-3255 ~ fax +39-06-6889-7689

# GROUND IMPACT UNCERTAINTY WINDOW

Currently BeppoSAX is approaching the reentry into the densest layers of the earth atmosphere. The expected residual lifetime of the satellite was computed by propagating its last available state vector (epoch: 29 April 2003, 18:52 UTC) with a numerical trajectory predictor including all the relevant perturbations. Concerning the predicted solar flux at 10.7 cm and the geomagnetic planetary index Ap, the last 3-day forecast, issued by the NOAA Space Environment Center, was adopted.

Using for the final orbits of BeppoSAX the ballistic parameter estimated by fitting the altitude decay observed during 19 hours, the ground impact time window shown in Table 1 was obtained for the main debris. The confidence level associated with such a window is approximately 90%.

Table 1

Ground Impact Time Window for the BeppoSAX Main Debris

| Impact Window | Impact Date | Impact Time |
| --- | --- | --- |
| Opening | 29 April 2003 | 22:48 UTC |
| Nominal Impact | 29 April 2003 | 23:36 UTC |
| Closure | 30 April 2003 | 00:57 UTC |

# AREAS OF THE PLANET POTENTIALLY INVOLVED

Due to the current uncertainty on the reentry time, significant areas of the equatorial belt in between 4.36 degrees North and South might still be affected by the impact of the surviving fragments of BeppoSAX. The sub-satellite ground tracks potentially at risk are shown in Figure 1.

Fig. 1 – Sub-satellite tracks potentially at risk during the

impact time window given in Table 1

The countries or territories that might still be potentially hit by the BeppoSAX falling fragments are:

- Africa: Burundi, Congo, Democratic Republic of Congo, Gabon, Kenya, Rwanda, Seychelles, Tanzania;

- Asia: Indonesia, Maldives;

- Oceania: Federated States of Micronesia, Kiribati, Palau;

- South America: Brazil, Colombia, Guyana, Venezuela.

The following countries or territories of the equatorial belt in between 4.36 degrees North and South are instead not included anymore – with a confidence level of 90% or higher – in the areas at risk for the BeppoSAX falling fragments: Cameroon, Central African Republic, Equatorial Guinea, Ethiopia, Sudan, Uganda, Cap Palmas, at the border between Liberia and Côte d’Ivoire, the Niger river delta, in Nigeria, Brunei, São Tomé and Príncipe, Nauru, French Guiana, Suriname, Somalia, Malaysia, Singapore, Marshall Islands, Papua New Guinea, Ecuador, Peru, Baker Island (USA), Howland Island (USA), Jarvis Island (USA).

# LONGITUDINAL RISK TIME WINDOWS

Table 2 lists the impact risk time windows, as a function of longitude, for the sub-satellite ground tracks potentially at risk, shown in Figures 1 and 2-5.

Table 2

Impact Risk Time Windows (UTC) Along the Risky Ground Tracks

| Longitude Band | Track No. 1 | Track No. 2 |
| --- | --- | --- |
| 180° – 120° W |  | 29 APR 2323 30 APR 0018 |
| 120° – 60° W |  | 29 APR 2338 30 APR 0033 |
| 60° W – 0° |  | 29 APR 2353 30 APR 0048 |
| 0° – 60° E | 29 APR 2236-2331 | 30 APR 0008-0104 |
| 60° – 120° E | 29 APR 2251-2347 | 30 APR 0024-0120 |
| 120° – 180° E | 29 APR 2307 30 APR 0003 | 30 APR 0040-0136 |

# CONTINENTAL RISK MAPS

The sub-satellite ground tracks at risk are presented in further detail, for specific continental areas, in the following maps (Figures 2-5), where a ground swath half width of 60 km was assumed as an example.

Fig. 2 – Sub-satellite track potentially at risk over South America

No. 2 of Table 2

Fig. 3 – Sub-satellite tracks potentially at risk over Africa and Maldives

Northern track: No. 1 of Table 2

Southern track: No. 2 of Table 2

Fig. 4 – Sub-satellite tracks potentially at risk over Southeast Asia

Northern track: No. 2 of Table 2

Southern track: No. 1 of Table 2

Fig. 5 – Sub-satellite tracks potentially at risk over Oceania

Northern track: No. 2 of Table 2

Southern track: No. 1 of Table 2

# NEXT UPDATE

The next bulletin will be issued as soon as new significant results are available.

