## No. 25                                                                                             30 April 2003 – 09:30 UTC

BEPPOSAX REENTRY REPORT

Prepared by Luciano Anselmo and Carmen Pardini

Space Flight Dynamics Laboratory

Istituto di Scienza e Tecnologie dell’Informazione “Alessandro Faedo”

CNR - Area della Ricerca di Pisa, Via Moruzzi 1, 56124 Pisa, Italy

ASI Contract No. I/408/02/0

BeppoSAX Reentry Situation Room: bepposax@protezionecivile.it

tel. +39-06-6804-3255 ~ fax +39-06-6889-7689

# BEPPOSAX REENTRY

Using for the final orbits of BeppoSAX the ballistic parameter estimated by considering the altitude decay observed during 7.3 hours (last couple of two line orbital elements available), the ground impact time window shown in Table 1 was estimated for the main debris. Figures 1 and 2 show the sub-satellite ground track corresponding to the above mentioned decay window. A ground swath half width of 60 km was assumed.

Table 1

Ground Impact Time Window for the BeppoSAX Main Debris

| Impact Window | Impact Date | Impact Time |
| --- | --- | --- |
| Opening | 29 April 2003 | 21:45 UTC |
| Nominal Impact | 29 April 2003 | 22:43 UTC |
| Closure | 29 April 2003 | 23:43 UTC |

The nominal reentry location, based on the American Space Surveillance Network assessment, is shown in the figures. It is affected by an uncertainty of plus or minus 3000 km along the trajectory. No visual report concerning the satellite reentry time and area is at present available.

Fig. 1 – Sub-satellite track corresponding to the BeppoSAX decay window

Fig. 2 – BeppoSAX reentry area