## No. 6                                                                                                                  17 March 2003

BEPPOSAX REENTRY PREDICTIONS

Istituto di Scienza e Tecnologie dell’Informazione “Alessandro Faedo”

CNR - Area della Ricerca di Pisa, Via Moruzzi 1, 56124 Pisa, Italy

ASI Contract No. I/408/02/0

Technical Manager: Carmen Pardini [Carmen.Pardini@cnuce.cnr.it]

Contact Point: Luciano Anselmo [Luciano.Anselmo@cnuce.cnr.it]

tel. +39-050-315-2952 ~ fax +39-050-313-8091

# SATELLITE ORBIT STATUS

Currently, the BeppoSAX satellite completes a revolution around the earth every 91.4 minutes at an altitude in between 338.8 km (perigee) and 340.5 km (apogee). The orbit is inclined by 3.96 degrees with respect to the equator, which means the satellite may fly over any location included in the latitude band between 3.98 degrees North and South. However, the motion is significantly perturbed by the residual atmosphere present at that altitude. The main effect of the resulting drag force is a progressively increasing altitude loss, which will culminate in the satellite reentry on the earth.

# BALLISTIC PARAMETER ESTIMATION

The nominal ballistic parameter BN, defined according to the relationship BN  CDA/m, where CD, m and A are, respectively, the satellite drag coefficient, mass and average cross-sectional area, was estimated by fitting, in a least squares sense, the satellite semi-major axis decay observed during the last two months. All the relevant orbit perturbations were included and the Jacchia-Roberts 1971 model was adopted to describe the varying atmospheric density. The orbital data were provided by the NASA/GSFC Orbital Information Group and by Nicholas L. Johnson of NASA/JSC. The value obtained was BN  0.02253 m2/kg.

# BEPPOSAX REENTRY WINDOW

The expected residual lifetime of the satellite was computed by propagating its last available state vector (epoch: 17 March 2003, 01:41 UTC)  with a numerical trajectory predictor including the geopotential harmonics up to the 16th order and degree, the luni-solar attraction, the solar radiation pressure with eclipses and the aerodynamic drag. The air density was computed with the Jacchia-Roberts 1971 model.

Concerning the predicted solar flux at 10.7 cm (used by the atmospheric model as a proxy of the solar irradiance at extreme ultraviolet frequencies) and the geomagnetic planetary index Ap, the last 27-day forecast (corresponding to one solar rotation), issued by the NOAA Space Environment Center, was adopted. For the subsequent months, such solar flux predictions were reiterated, applying an average reduction of about five standard units per month, while a constant average value (Ap  12) was used for the geomagnetic planetary index.

Using for BeppoSAX the ballistic parameter estimated by fitting the orbital decay observed during the last two months, the nominal reentry date of 2 May 2003 was obtained. In order to take into account the intrinsic uncertainties of this kind of predictions, a reentry window with an assumed confidence level of approximately 90% was computed by varying the nominal ballistic parameter BN of  25%, as shown in Table 1.

Table 1

BeppoSAX Reentry Window

| Reentry Window | Criteria | B (m2/kg) | Reentry Date |
| --- | --- | --- | --- |
| Opening | BN + 25% | 0.02816 | 22 April 2003 |
| Nominal Reentry | BN | 0.02253 | 2 May 2003 |
| Closure | BN  25% | 0.01690 | 19 May 2003 |

At present, the solar flux predictions are still the leading source of uncertainty. This situation will change only a few weeks before reentry, when the ballistic coefficient will play a comparable role. Geomagnetic storms might have a significant impact on the reentry date during the last ten days of satellite lifetime.

# NEXT UPDATE

The next reentry prediction will be issued no later than 27 March 2003.

 Prepared by Luciano Anselmo and Carmen Pardini 